Ethical Hacking Lab

Module 2:

Scanning networks, Enumeration and sniffing

A. Port Scanning

Aim: To identify open ports and services running on a target system

Theory: Port Scanning

Port scanning is a technique used to discover open ports and associated services on a networked device. Ports are endpoints for network communications and each port can be used by different services or applications.

Command:

Using cmd

> nmap 192.168.56.1

> nmap -p 22,80,443 192.168.56.1

> nmap -p 1-65535 192.168.56.1

Using Nmap - Zenmap GUI

> nmap -T4 -A -v 192.168.56.1

Output:

Download Nmap: https://nmap.org/download.html

B. Network Scanning Tools

IP Addresses: 45.33.49.119; 142.250.182.196 & 54.182.0.79

Aim: To use network scanning tools, specifically Nmap with NPCAP, to scan multiple IP addresses for open ports and services

Theory: Network Scanning Tools

Network scanning tools like Nmap, when used with packet capture libraries such as NPCAP, are crucial for network assessment and security analysis. Nmap is used to discover hosts and services on a network by sending packets and analyzing responses. NPCAP enhances Nmap's capabilities by providing real-time packet capture on Windows systems. Scanning multiple IP addresses helps in identifying which ports are open and which services are running, offering insights into the network's security and configuration.

Command:

Using Nmap - Zenmap GUI

> nmap -T4 -A -v 45.33.49.119 | > nmap -T4 -A -v 142.250.182.196 | > nmap -T4 -A -v 54.182.0.79

Output:

C. IDS Tool

Aim: To deploy and configure an Intrusion Detection System (IDS) tool for monitoring and detecting potential security breaches in a network

Theory: IDS Tool

An Intrusion Detection System (IDS) is a crucial security tool used to monitor network traffic for suspicious activities and potential threats. IDS tools analyze the incoming and outgoing traffic and compare it against known attack patterns or signatures. When an anomaly or a match is detected, the IDS generates alerts, enabling administrators to take appropriate action to mitigate risks. IDS can be categorized into two types: Network-based IDS (NIDS) and Host-based IDS (HIDS). NIDS monitors traffic across an entire network segment, while HIDS focuses on individual host activities. IDS systems can operate in real-time, providing timely alerts, and are an integral part of a comprehensive cybersecurity strategy, helping organizations safeguard against unauthorized access, data breaches, and other malicious activities.

Steps to be followed:

- Download Snort from the snort.org website.

- Download Rules from the snort.org website. You must register to get the rules (you should download these often): snortrules-snapshot-29200.tar.gz

- Install Snort. Double-click the .exe file to install Snort. This will install Snort in the C:\Snort folder. It is important to have npcap or WinPcap installed.

- Extract the Rules file. You will need WinRAR for the .gz file.

- Copy all files from the rules folder of the extracted folder. Now paste the rules into the C:\Snort\rules folder.

- Copy the snort.conf file from the etc folder of the extracted folder. Paste it into the C:\Snort\etc folder. Overwrite any existing file. Remember, if you modify your snort.conf file and download a new one, you must modify it for Snort to work.

- Open a command prompt (cmd.exe) and navigate to the C:\Snort\bin folder. (At the prompt, type cd\snort\bin)

- To start (execute) Snort in sniffer mode, use the following command: snort -dev -i 7

  - -i interface specifies which interface Snort should listen on. This option is used on machines that have more than one network interface card or different kinds of interfaces besides Ethernet.

  - -dev is used to run Snort to capture packets on your network.

- To check the interface list, use the following command: snort -W

- You can tell which interface to use by looking at the Index number and finding Microsoft. As you can see in the example, the other interfaces are for VMware.

- To run Snort in IDS mode, you will need to configure the snort.conf file according to your network environment.

- To specify the network address that you want to protect in the snort.conf file, look for the following line: var HOME_NET 192.168.1.0/24 (You will normally see any here). Find your system IP Address.

- You may also want to set the addresses of DNS_SERVERS if you have some on your network. Example:

- Change the RULE_PATH variable to the path of the rules folder: var RULE_PATH c:\snort\rules

- Change the path of all library files with the name and path on your system. You must change the path of the snort_dynamicpreprocessor variable: C:\Snort\lib\snort_dynamicpreprocessor. You need to do this to all library files in the C:\Snort\lib folder. The old path might be: /usr/local/lib/.... You will need to replace that path with your system path using C:\Snort\lib.

- Change the path of the dynamicengine variable value in the snort.conf file. Example: dynamicengine C:\Snort\lib\snort_dynamicengine\sf_engine.dll

- Add the paths for include classification.config and include reference.config files

  - include c:\Snort\etc\classification.config

  - include c:\Snort\etc\reference.config

- Remove the comment (#) on the line to allow ICMP rules, if it is commented with a #:

  - include $RULE_PATH/icmp.rules

- You can also remove the comment on ICMP-info rules if it is commented:

  - include $RULE_PATH/icmp-info.rules

- To add log files to store alerts generated by Snort, search for the output log test in snort.conf and add the following line:

  - output alert_fast: snort-alerts.ids

- Comment (add a #) to the whitelist: $WHITE_LIST_PATH/white_list.rules and the blacklist. Change the nested_ip inner , \ to nested_ip inner #, \

- Comment out (#) the following lines:

  - #preprocessor normalize_ip4

  - #preprocessor normalize_tcp: ips ecn stream

  - #preprocessor normalize_icmp4

  - #preprocessor normalize_ip6

  - #preprocessor normalize_icmp6

- Save the snort.conf file.

- To start Snort in IDS mode, run the following command:

  - snort -c c:\snort\etc\snort.conf -l c:\snort\log -i 3

  - If a log is created, select the appropriate program to open it. You can use WordPad or Notepad++ to read the file. If you encounter an error on line 324, comment decompress_swf { deflate lzma } \ and replace it with decompress_swf { deflate } \ and decompress_pdf { deflate }.

Command:

C:\Snort\bin>

> cd C:\Snort\bin | > snort -V | > snort -dev -i 5 | > snort -W | > snort -i 5 -c C:\Snort\etc\snort.conf -T

Output:

Download Snort: https://snort.org/downloads

D. Sniffing Tool

Aim: To use a network sniffing tool to capture and analyze network traffic for monitoring and identifying suspicious activities

Theory: Sniffing Tool

Network sniffing tools are designed to capture data packets as they travel across a network, allowing administrators to monitor, analyze, and troubleshoot network issues. These tools can help identify potential security threats, such as unauthorized access or data leakage, by providing insights into the type of traffic, the source and destination of packets, and the nature of the data being transmitted. Sniffing tools can operate in both passive and active modes, where passive sniffing simply monitors the traffic without interference, while active sniffing may involve injecting packets into the network. Effective use of these tools is essential in maintaining network security and ensuring the integrity of data communication.

Steps to be followed:

- Download Wireshark from the official website of wireshark.

- Open Wireshark. You will get the following screen.

- Select the network interface you want to sniff. Note: for this demonstration, we are using a wireless network connection. If you are on a local area network, then you should select the local area network interface. Click on the start button as shown above.

- Open your web browser and type in http://www.techpanda.org/

- The login email is admin@google.com and the password is Password2010.

- Click on the submit button.

- A successful logon should give you the following dashboard.

- Go back to Wireshark and stop the live capture.

- Filter for HTTP protocol results only using the filter textbox.

- Locate the Info column and look for entries with the HTTP verb POST and click on it.

- Just below the log entries, there is a panel with a summary of captured data. Look for the summary that says Line-based text data: application/x-www-form-urlencoded.

Search:

1. Open the website:

- Go to http://www.techpanda.org/ in your web browser.

2. Locate the login section:

- Find the fields where you can input your email and password. This is usually on the homepage or under a "Login" or "Sign In" section.

3. Enter the email:

- Click on the email field.

- Type admin@google.com.

4. Enter the password:

- Click on the password field.

- Type Password2010.

5. Submit:

- Click the "Submit" button or the equivalent action button to log in.

Output:

Download Wireshark: https://www.wireshark.org/download.html